1. What is a Network?

Definition

A network is a group of connected devices that communicate and share resources.

Real World Examples

- Public transportation systems

National power grids

- Postal systems

- Social connections

Computing Examples

- Computers

- Phones

- Servers

- Cameras

- IoT devices

- Traffic systems

Key Point

Networks can range from:

- 2 devices
to

- Billions of devices

2. The Internet

Definition

The Internet is a global network made up of many smaller networks.

Network Types

| Type | Description |
| --- | --- |
| Private Network | Internal/local network |
| Public Network | Internet/global network |

3. History of the Internet

| Year | Event |
| --- | --- |
| 1960s | ARPANET created |
| 1989 | World Wide Web invented |

WWW invented by:
Tim Berners-Lee

4. Device Identification

Devices use two identifiers:

| Identifier | Purpose |
| --- | --- |
| IP Address | Logical identification |
| MAC Address | Physical identification |

5. IP Address

Definition

An IP address identifies a device on a network.

IPv4 Structure

192.168.1.1

- 4 octets

- Range per octet: 0–255

6. Public vs Private IP

| Type | Usage |
| --- | --- |
| Private IP | Local network communication |
| Public IP | Internet communication |

Example

| Device | Private IP | Public IP |
| --- | --- | --- |
| PC1 | 192.168.1.77 | 86.157.52.21 |
| PC2 | 192.168.1.74 | 86.157.52.21 |

7. IPv4 vs IPv6

IPv4

Uses:
2^{32}

≈ 4.29 Billion addresses

Example:

192.168.1.1

IPv6

Uses:
2^{128}

Very large address space

Example:

2001:0db8:85a3:0000:0000:8a2e:0370:7334

Benefits

- More IP addresses

- Better efficiency

- Solves IPv4 shortage

8. MAC Address

Definition

A unique hardware address assigned to a network interface.

Example

a4:c3:f0:85:ac:2d

Structure

| Part | Meaning |
| --- | --- |
| First 6 characters | Manufacturer |
| Last 6 characters | Unique device ID |

9. MAC Spoofing

Definition

Changing/faking a MAC address to impersonate another device.

Risk

Can bypass weak security controls.

10. Ping

Definition

A network utility used to test connectivity.

Uses:

- ICMP protocol

Syntax

ping 8.8.8.8

11. ICMP

Full Form

Internet Control Message Protocol

Purpose

- Connectivity testing

- Network diagnostics

12. LAN Topologies

Definition

Topology = layout/design of a network.

13. Star Topology

Structure

All devices connect to a central switch/hub.

PC1

|

PC2 --SWITCH-- PC3

|

PC4

Advantages

- Reliable

- Scalable

- Easy device addition

Disadvantages

- Expensive

- Central device failure affects all

14. Bus Topology

Structure

All devices share one backbone cable.

PC1 -- PC2 -- PC3 -- PC4

Advantages

- Cheap

- Easy setup

Disadvantages

- Bottlenecks

- Single point of failure

- Difficult troubleshooting

15. Ring Topology

Structure

Devices connected in a loop.

PC1 -- PC2

|      |

PC4 -- PC3

Advantages

- Less cabling

- Fewer bottlenecks

Disadvantages

- One failure breaks network

- Slower routing path

16. Switch

Definition

A device that connects multiple devices in a LAN.

Functions

Sends traffic to correct device

- Reduces unnecessary traffic

Port Sizes

- 4

- 8

- 16

- 24

- 48

- 64

17. Router

Definition

A device that connects different networks.

Main Function

Routing packets between networks.

18. Routing

Definition

Process of determining the best path for data.

Factors

- Shortest path

- Reliability

- Fastest medium

19. Subnetting

Definition

Dividing a network into smaller subnetworks.

Benefits

- Efficiency

- Security

- Better management

20. Subnet Mask

Example

255.255.255.0

Used to identify:

- Network portion

- Host portion

21. Types of Addresses in Subnetting

| Type | Example | Purpose |
| --- | --- | --- |
| Network Address | 192.168.1.0 | Identifies network |
| Host Address | 192.168.1.100 | Identifies device |
| Default Gateway | 192.168.1.254 | Connects outside network |

22. ARP

Full Form

Address Resolution Protocol

Purpose

Maps:

- IP Address ↔ MAC Address

23. ARP Process

ARP Request

Who owns this IP?

ARP Reply

This MAC belongs to that IP

24. ARP Cache

Definition

Stores IP-to-MAC mappings temporarily.

25. DHCP

Full Form

Dynamic Host Configuration Protocol

Purpose

Automatically assigns IP addresses.

26. DHCP Process (DORA)

| Step | Meaning |
| --- | --- |
| Discover | Search DHCP server |
| Offer | Offer IP |
| Request | Request offered IP |
| ACK | Confirm assignment |

27. OSI Model

Definition

A 7-layer networking framework.

| Layer | Name |
| --- | --- |
| 7 | Application |
| 6 | Presentation |
| 5 | Session |
| 4 | Transport |
| 3 | Network |
| 2 | Data Link |
| 1 | Physical |

28. Physical Layer (Layer 1)

Responsibilities

- Physical transmission

- Electrical signals

Binary communication

Example

- Ethernet cable

29. Data Link Layer (Layer 2)

Responsibilities

- MAC addressing

- Frame transmission

Device

- Switch

30. Network Layer (Layer 3)

Responsibilities

- Routing

- IP addressing

Device

- Router

31. Transport Layer (Layer 4)

Responsibilities

- Data transport

- Reliability

- Error checking

Protocols

- TCP

- UDP

32. Session Layer (Layer 5)

Responsibilities

- Establish sessions

- Maintain connections

- Close sessions

33. Presentation Layer (Layer 6)

Responsibilities

- Translation

- Encryption

- Formatting

Example

- HTTPS encryption

34. Application Layer (Layer 7)

Responsibilities

User interaction with network services.

Examples

- Browsers

- Email clients

- DNS

35. Packets vs Frames

| Packet | Frame |
| --- | --- |
| Layer 3 | Layer 2 |
| Contains IP info | Contains MAC info |

36. Packet Headers

| Header | Purpose |
| --- | --- |
| TTL | Packet expiry |
| Checksum | Integrity check |
| Source Address | Sender IP |
| Destination Address | Receiver IP |

37. TCP

Full Form

Transmission Control Protocol

Features

- Reliable

- Connection-oriented

- Error checking

Uses

- File transfer

- Email

- Web browsing

38. TCP Three-Way Handshake

| Step | Message |
| --- | --- |
| 1 | SYN |
| 2 | SYN/ACK |
| 3 | ACK |

39. TCP Closing Connection

| Step | Purpose |
| --- | --- |
| FIN | Request close |
| ACK | Confirm close |

40. UDP

Full Form

User Datagram Protocol

Features

- Fast

- Connectionless

- No guarantee of delivery

Uses

- Streaming

- Voice chat

- Gaming

41. TCP vs UDP

| TCP | UDP |
| --- | --- |
| Reliable | Faster |
| Connection-oriented | Connectionless |
| Error checking | No guarantee |
| Slower | Faster |

42. Port Forwarding

Definition

Allows external devices to access internal services.

Example

Public users accessing internal webserver.

If the administrator wanted the website to be accessible to the public (using the Internet), they would have to implement port forwarding, like in the diagram below:

43. Firewall

Definition

Controls allowed and blocked network traffic.

Checks

- Source

- Destination

- Port

- Protocol

44. Firewall Types

Stateful Firewall

- Tracks entire connection

- More intelligent

- Resource intensive

Stateless Firewall

- Uses static rules

- Faster

- Less intelligent

45. VPN

Full Form

Virtual Private Network

Purpose

Creates encrypted tunnel over Internet.

46. VPN Benefits

| Benefit | Description |
| --- | --- |
| Privacy | Encrypts traffic |
| Security | Safe communication |
| Remote Access | Connect remote offices |
| Anonymity | Hides activity |

47. VPN Technologies

| Technology | Description |
| --- | --- |
| PPP | Authentication/encryption |
| PPTP | Easy setup |
| IPSec | Strong encryption |

48. Layer 2 vs Layer 3 Switch

| Layer 2 Switch | Layer 3 Switch |
| --- | --- |
| Uses MAC addresses | Uses IP + MAC |
| Frame forwarding | Routing + switching |

49. VLAN

Full Form

Virtual Local Area Network

Purpose

Virtually separates devices on same switch.

Benefit

Improves:

- Security

- Traffic management

- Segmentation